BỘ GIÁO DỤC VÀ ĐÀO TẠO

TRƯỜNG ĐẠI HỌC MỎ - ĐỊA CHẤT

BÁO CÁO HỌC THUẬT

FILM FLOW ON A ROTATING DISK

Pr. Brian G. Higgins,  T. S. Hoàng Ngự Huấn

Hà Nội, 12/2019

BỘ GIÁO DỤC VÀ ĐÀO TẠO

TRƯỜNG ĐẠI HỌC MỎ - ĐỊA CHẤT

BÁO CÁO HỌC THUẬT

FILM FLOW ON A ROTATING DISK

Xác nhận của bộ môn

Hà Nội, 5/2019

Contents

I)	Introduction	4

II. Mathematical formulation	7

III. Asymptotic analysis for small Re	10

IV. Short – time analysis for arbitrary Re	17

V. Discussion	22

VI. Concluding remarks	24

Acknowledgments	25

Articles you may be interested in	26

Brian G. Higgins

Department of Chemical Engineering, University of California, Davis, California 95616

(Received 9 May 1986; accepted 15 August 1986)

Unsteady liquid film flow on a rotating disk is analyzed  by asymptotic methods for low and high Reynolds numbers. The analysis elucidates how a film of uniform thickness thins when the disk is set in steady rotation. In the low Reynolds number analysis two time scales for the thinning film are identified. The long-time-scale analysis ignores he initial acceleration of the fluid layer and hence is singular at the onset of rotation. The singularity is removed by matching the long-time-scale expansion for the transient film thickness with a short-time-scale expansion that accounts for fluid acceleration during spinup. The leading order term in the long-time-scale solution for the transient film thickness is shown to be a lower bound for film thickness for all time. A short-time analysis that accounts for boundary layer growth at the disk surface is also presented for arbitrary Reynolds number. The analysis becomes invalid either when the boundary layer has a thickness comparable to that of the thinning film, or when nonlinear effects become important.

- Introduction

In this paper we present an asymptotic solution to the unsteady Navier-Stokes equations for film flow on a rotating disk. We restrict attention to film flow with a planar interface and assume that the disk radius is much larger than the film thickness so that edge effects are confined to a small region of the total film area and can therefore be neglected. The work of Emslie et al., as discussed below, suggests that assumption of a planar interface is not unreasonable. A principal feature of the solution is that the radial and azimuthal components of velocity assume a linear dependence to be factored out of the governing equations and boundary conditions. The pressure field, independent of radial position, can then be expressed in terms of the axial component of velocity induced by the thinning film. The principal objective of this paper is to derive an asymptotic solution to the resulting nonlinear, time-dependent boundary value problem in the limit of vanishingly small Reynolds number. However, a short-time analysis for arbitrary Reynolds number is also undertaken to clarify the effects of viscosity on how the film thins after the disk is impulsively started from rest.

This work is motivated in part by our interest in studying centrifugal spinning of liquid films as a convenient method for coating a discrete planar substrate, such as a disk, with a uniform film. In the coating literature this technique is referred to as spin coating, and is widely used in the manufacture of integrated circuits, magnetic and optial disks for data storage, color television screens, and optical mirrors. In many applications the coating liquid is initially applied to the disk as a thick layer, which is then thinned by rapidly spinning the disk. The final film thickness and its uniformity are known to depend on several process parameters, most notably the rotational speed, liquid viscosity, and, when solvents are present, the rate of solvent evaporation.

The first detailed hydrodynamic analysis of spin coating of a Newtonian liquid was given by Emslie et al. They assumed that the local contrifugal force per unit volume (given by ρrΩ2, where Ω is the angular velocity of the disk) is uniform across the thinning film and is balanced solely by viscous shear across the film due to the radial liquid flux. The pressure is taken to be everywhere uniform and inertial forces other than centrifugal are assumed to be unimportant. As noted by Acrivos et al., these assumptions are expected  to be valid when the Reynolds number Re=h02Ω/υ is much less than unity (h0 being a characteristic length scale for the thinning film, and υ the kinematic viscosity). Emslie et al. were able to show that initial irregularities in the film profile would tend to smooth out with spinning, and a fluid layer that was initially uniform would maintain its uniformity with spinning. These predictions are exclusively for Newtonian liquids and are not necessarily valid for non-Newtonian liquids. For instance, Acrivos et al. have shown that a uniform film can not, in general, be obtained by spinning a power-law liquid.

In recent years sereval mathematical models for spin coating of photoresists have been proposed in the literature. Although additional effects, such as solvent evaporation and non-Newtonian behavior, have been accounted for in the analyses, the hydrodynamic approximations used are typically  those employed by Emslie et al. To the best of our knowledge there have been no attempts in the literature to develop a rigorous theory for the hydrodynamics of spin coating that goes beyond the analysis of Emslie et al.

In Sec. II we state the mathematical problem, and introduce the form for the velocity field that allows the r dependence to be factored out of the Navier-Stockes equations and boundary conditions. We show that it is possible then to derive a general expression for the pressure field in terms of he velocity component normal to the disk surface.

Section III is devoted to the asymptotic analysis in the limit of vanishingly small Reynolds number. We develop an analytic solution based on a long-time-scale expansion. This solution does not satisfy the initial conditions and must be matched, in an asymptotic sense, to a short-time-scale solution. The mathematical problem discussed in Sec. III is similar in sereval aspects to the low Reynolds number solution for the time-dependent draining of a thin fluid layer between parallel plates under a constant applied force recently discussed by  Weinbaum et al. In our problem the driving force is centrifugal acceleration. The other significant difference  is the additional independent equation required for the azimuthal component of velocity.

In Sec. IV a short-time analysis is given for the boundary layer flow that is formed after the disk is impulsively started from rest. This analysis, valid for all Re, draws heavily on the work of Lawrence et al. In a follow-up paper to Ref. 9 these authors consider the boundary layer formation that occurs when a thin fluid layer is suddenly drained between parallel plates under a constant applied force. We show that the govering equations that describe boundary layer growth for our problem are to leading order identical to those given by Benton. He analyzed  flow due to a rotating disk started impulsively from rest in an infinite expanse of fluid. By matching the axial velocity in the boundary layer with its counterpart in the overlying inviscid layer, we are able to derive an expression for how the film thins with time. The asymptotic solution are discussed in Sec. V and their relation to the work of Emslie et al. displayed. There are some similarities between our work at arbitrary Re and the time-dependent spinup of a rotating fluid analyzed by Greenspan and Howard. For a discussion of these similarities readers are referred to Benton’s article.

# II. Mathematical formulation

We consider a uniform liquid film on a horizontal plane disk of arbitrary large diameter. This disk is made to rotate in its own plane with a steady angular velocity Ω. We restrict our attention to flows that maintain a film thickness that is independent of the radial coordinate r, and azimuthal coordinate θ. The coordinate axes as well as the general dimentions of the flow are indicated in Fig. 1. The initial film thickness is h0.

We take the fluid velocity in the thinning film to be independent of θ:

v=ur,z,ter+vr,z,teθ+wr,z,tez, (1)

where er,eθ,ez are the unit vectors along the coordinate axes. The equations of continuity and motion for an incompressible Newtonian fluid satisfying (2.1) are

∂u∂r+ur+∂w∂z=0, (2a)

∂u∂t+u∂u∂r-v2r+w∂u∂z=-1ρ∂p∂r+v∂2u∂r2+∂∂rur+∂2u∂z2, (2b)

∂v∂t+u∂v∂r+uvr+w∂v∂z=v∂2v∂r2+∂∂rvr+∂2v∂z2, (2c)

∂w∂t+w∂w∂z+u∂w∂r=-1ρ∂p∂z+v∂2w∂r2+1r∂w∂r+∂2w∂z2, (2d)

where v is kinematic viscosity of the liquid, ρ the density, and p is the pressure. The boundary conditions that (2) must satisfy are as follows. At the surface of the rotating disk the adherence and impenetrability conditions stipulate that

ur,0,t=0,     vr,0,t=Ωr,     wr,0,t=0. (3)

At the free surface z=ht the normal and tangential components of the traction vector must vanish:

-p+2μ∂w∂z=0, (4a)

∂w∂r+∂u∂z=0,      ∂v∂z=0. (4b)

In addition, the kinematic condition dictates that the instantaneous velocity of the free surface and the z component of the fluid velocity be equivalent:

dhdt=wr,h,t. (5)

The initial conditions for motion in the liquid film are

v=0,     h=h0,     dhdt=0,    at   t=0 (6)

Equations (1) –  (6) define a time-dependent, two-dimensional free boundary value problem.

We will seek a solution to (1) – (6) of the form (cf. von Karman)

u=rfz,t,   v=rgz,t,  w=wz,t. (7)

Substituting (7) into the continuity equation (2a) provides a relation between f and w:

2f+wz=0. (8)

(In what follows we will use a subscript to denote partial differentiation, i. e., wz≡∂w/∂z.) The r component of momentum in terms of (7) becomes

ft+f2-g2+wfz-vfzz=-1/ρ1/rpr. (9)

It is evident that the r dependence can be factored out of (9) by defining the pressure to be

pr,z,t=-ρr2/2Az,t+Bz,t. (10)

The quantities Az,t and Bz,t are unknown functions of integration. The w component of momentum in  term of (7) and (10) becomes

ρwt+wwz-vwzz=ρr2/2Az-Bz. (11)

Thus in order for the right-hand side of (11) to be independent of r, we must have Az=0. Moreover, since the viscous term in the normal stress boundary condition (4a) is independent of r, this implies that Az,t=0. Integrating (11) with respect to z from z to z=ht gives an expression for Bz,t:

Bz,t=Bh,t+ρzhwt dz+ρw22-vwzzh. (12)

The boundary term Bh,t can be evaluated with (4a). Hence, the pressure in the thinning film is given by:

pz,t=ρvwz+w22z=ht-ρw22-vwz-zhtwt dz. (13)

Finally, the r dependence can be factored out of the θ component of momentum to give

gt+2fg+wgz=vgzz. (14)

The boundary conditions for f and g follow dirctly from (3) and (4):

ft,0=0,    gt,0=Ω,    wt,0=0, (15)

fzt,h=0,     gzt,h=0, (16a)

ht=wt,h. (16b)

The initial conditions for f and g become

f0,z=0,      g0,z=0,      w0,z=0, (17)

and

h=h0,      ht=0,   at   t=0.  (18)

The Navier – Stokes equations (2) have been reduced to a coupled set of three partial differential equations for f, g and w, viz. (8), (9), and (14), subject to the boundary and initial conditions given by (15) – (18). The kinematic condition (16b) provides the additional equation for determining the film thickness. Interestingly, the pressure field, consequence of normal viscous stresses and inertia of the axial component of velocity induced by the thinning film, can be determined from (13) once w and h are known, and thus need not be solved for simultaneously with the other dependent variables.

Although the analysis presented so far is strictly correct for a flat interface, the more complicated case of a nonuniform interface h=hr,t can be analyzed in a similar manner in the limit of zero capillary number (infinite surface tension) by the technique of domain perturbations (Joseph). In the limit of zero capillary number the selection of  a uniform interface as a base state leads to a sequence of problems involving cylindrial coordinates in which the r dependence can be factored out of the perturbation equations. Of course, higher-order corrections to the pressure field will now depend on r as well as z. The case of a nonuniform interface will be considered in future work.

# III. Asymptotic analysis for small Re

When the Renolds number is much less than unity we anticipate that the centrifugal force [g2 in (9)] will be of comparable magnitude o viscous shear across the film induced by the radial liquid flux (cf. Emslie et al.). A balance between viscous shear and centrifugal force may be used to determine a characteristic time scale tc for the flow. To proceed with the analysis we introduce the following dimensionless variables:

τ=ttc,   ξ=zh0,    H=hh0,    F=ftc,

W=wtch0,      G=gΩ. (19)

On substituting these variables into (8), (9) and (14) and then requiring in the limit of vanishingly small Reynolds number (Re≡h02Ω/v) that the centrifugal force be balanced by viscous shear across the film, we obtain the appropriate expression for the time scale tc:

tc=v/h02Ω2. (20)

The meaning of this time scale becomes clear when one recalls that the time for vorticity generated at the disk boundary to spread through the film on spinup is h02/v, whereas the time scale for inertial spining is 1/Ω. Thus tc may be interpreted as the time required after spinup for the film to thin in the absence of all inertial forces other than centrifugal.

In terms of the dimensionless variables (19) and (20) the boundary value problem for F, G, W and H becomes

2F+Wt=0, (21a)

Re2Fτ+F2+WFt-G2=Ftt (21b)

Re2Gτ+2FG+WGt=Gtt, (21c)

With boundary conditions

Fτ,0=0,    Gτ,0=1,    Wτ,0=0,

Fξτ, H=0,    Gξτ, H=0,     Hτ=Wτ, H, (22)

And initial conditions

F0,ξ=0,    G0,ξ=0,    W0,ξ=0,

H=1,    Hr=0. (23)

To construct an asymptotic solution for Re≪1, we assume the following forms for the expansions of the dependent variables:

Fξ,τ;Re~F0ξ, H+Re2F1ξ, H, Hτ+oRe4,

Gξ,τ;Re~G0ξ,H+Re2G1ξ, H, Hr+oRe4,

Wξ,τ;Re~W0ξ, H+Re2W1ξ, H,Hτ+oRe4,

Hτ;Re~H0τ+Re2H1τ+oRe4.

As we will show shortly these expansions are not valid for all time. In particular they are singular at τ=0, and thus represent the long – time – scale solution. A short – time – scale solution will be required to satisfy the initial conditions.

In the limit Re→0 with τ and ξ fixed, the zero-order problem becomes

2F0+W0ξ=0, (25a)

F0ξξ+G02=0, (25b)

G0ξξ=0, (25c)

With boundary conditions

F0τ,0=0,    G0τ,0=1,    W0τ,0=0,

F0ξτ, H=0,    G0ξξ, H=0. (26)

Since we have lost the local inertial terms in the zero-order problem, the initial conditions on the velocity field must be discarded at this level of apppoximation. The solution to (25) satisfying (26) is

F0ξ,H=-ξ2/2+Hξ,     G0ξ, H=1,

W0ξ, H=ξ3/3-Hξ2. (27)

The first-order problem at oRe2 is

2F1+W1ξ=0,(28a)

F1ξξ=F0τ+F02+W0F0ξ-2G0G1, (28b)

G1ξξ=Goτ+2F0G0+W0G0ξ, (28c)

with boundary conditions

G1τ,0=0,    F1τ,0=0,      W1τ,0=0,

G1ξτ,H=0,       F1ξτ, H=0. (29)

Because the right – hand side of (28c) is known from the zero-order problem, (28c) can be integrated to find

G1=-ξ412+Hξ33-2H3ξ3. (30)

The expression for G1 together with (27) can be substituted into (28b) to give

F1ξξ=Hτξ+ξ412-Hξ33+43H3ξ. (31)

Integrating twice and applying (29) we obtain

F1=Hτξ36-H2ξ2+ξ6360-Hξ560+29H3ξ3-35H5ξ. (32)

Finally, from the continuity equation (28a) and boundary condition (29) we obtain the expression for W1:

W1=HrH2ξ22-ξ412-ξ71260+Hξ6180-19H3ξ4+35H5ξ2. (33)

The kinematic condition at (22) provides the additional relation needed to determine the film thickness as a function of time:

Hτ~W0H,τ+Re2W1H, τ=-23H3+Re2512HτH4+311630H7. (34)

Substituting the asymptotic expansion for H [see (24)] into (34) and separating powers of Re yields the following differential equations for H0 and H1:

H0τ=-23H03, (35a)

H1τ=-2H0H12+512H0τH04+311630H07. (35b)

These equations are readily integrated to give

H0=34τ+C01/2,

H1=C134τ+C03/2-1710534τ+C05/2. (36)

The constants of integration C0 and C1 are to be determined from the initial conditions (23). Because the velocity components given by the expansions for F, G, and W do not satisfy the initial conditions, an “inner” expansion valid for short times is required, which must be matched to the long – time – scale solution (36) to determine C0 and C1.

Short – time – scale analysis: At short time scales the local inertial terms in (21) are of the same order of magnitude as he viscous and centrifugal terms included in the long – time – scale analysis. In order to reflect this, we must introduce a nem time scale that characterizes  the time required for vorticity to diffuse throught the film, i. e., a time scale for order h02/v. The appropriate dimensionless variables for the short – time – scale are

T=tΩRe=τRe2,      F=fΩRe=F,       W=wh0ΩRe=W,

H=h/h0=H,  η=z/h0=ξ. (37)

Note in the short – time – scale analysis only the time variable is stretched. Substituting these variables into (21) and (22) gives

2F+Wη=0, (38a)

FT+Re2F2+WFη-G2=Fηη, (38b)

GT+Re22FG+WGη=Gηη, (38c)

with boundary conditions

FT, 0=0,            GT, 0=1,           WT,0=0,

FηT, H=0,        GηT,H=0,         HT=Re2WT, H. (39)

The initial conditions is terms of the short – time – scale variables are

F0,η=0,     G0,η=0,     W0,η=0,

H=1,     HT=0. (40)

The zero – order problem for the short – time – scale solution in the limit Re→0 with T and η held fixed follows by setting all terms multiplied be Re in (38) – (40) to zero. This yields a linear set of partial differential equations. An important consequence of the limiting process is that at zero order the film thickness can be determined directly from the kinematic condition at (39). The result is

H0T=0    or    H0=1. (41a)

(Here, as before, a subscript zero or one on a dependent variable denotes the order of the approximation). Thus at the zero – order appromation the axial coordinate η is specified in the range 0≤η≤1.

The govering equations for G0 follow directly form (38) – (40):

G0T=G0ηη,     0≤η≤1,   T>0,

G0T,0=1,     G0TT,1=0,  G00,η=0. (41b)

Equations (41) admit a separable solution given by

G0=1-2n>0Bnη,T;λn, (42)

where

Bn≡sinλnηλn e-λn2T  and  λn=nπ2,      n=1,3,5. (43)

The zero – order problem for F0 is

F0T-F0ηη=G02,         0≤η≤1,       T>0,

F0T,0=0,      F0ηT, 1=0,     F00,η=0. (44)

Equations (44) are conveniently solved by finite Fourier transforms. If we define the sine transforms for F0 and G02:

F≡01F0T,ηsinλmηdη, (45a)

G≡01G02T,ηsinλmηdη, (45b)

with λm=mπ/2, and m odd, then (44) becomes

dFdT+λm2F=G,     F0=0. (46)

The general solution to (46) is

F=e-λm2T0TGeλm2tdT, (47)

and after inverting the transform for F, we obtain

F0T,η=2m>0sinλmηe-λm2T0TGeλm2tdt. (48)

To proceed further it is necessary to evaluate G. From (43) we can write the transform for G02 as

G=011-4m>0Bn+4n>0Bn2sinλmηdη. (49)

The infinite series in the integrand of (49) can be integrated term by term to yield

G=1λm1-2e-λm2T-8n>0Anm0e-2λn2T-16p>0n>pAnmpe-λp2+λn2T, (50)

where

Anm0≡1λmλm2-4λn2,

Anmp≡λpλn2-λm22+λp4-2λp2λn2+λm2. (51)

The subscripts n,m and p are summed over all old positive intergers. Substituting the above expression for G into (48) and integrating with respect to time yields the required expression for F0T,η:

F0T,η=η-η22-2m>0sinλm

F0T,η=η-η22-2m>0sinλmηλm3e-λm2T-4m>0sinλmηλmTe-λm2T+16m>0sinλmηn>0Anm02λn2-λm2e-2λn2T-e-2λm2T+32m>0sinλmηp>0n>pAnmpλp2+λn2-λm2e-λp2+λn2T-e-λm2T.

Note, in the derivation of (52) we have used the result that

η-η22=2m>0sinλmηλm3. (53)

Once F0 is found, the continuity equation (38a) can be integrated to find W0:

W0T,η=-20ηF0dη=η33-η2-4m>01λm4cosλm,η-1e-λm2T-8m>01λm2cosλmη-1Te-λm2T+32m>01λmcosλmη-1n>0Anm02λn2-λm2e-2λn2T-e-λm2T+64m>01λmcosλmη-1p>0n>pAnmpλp2+λn2-λm2e-λp2+λn2T-e-λm2T. (54)

This completes the short – time – scale solution at zero order. However, to determine the constants C0 and C1 in the long – time – scale solution (36), it is necessary to compute the film thickness for the short – time – scale solution to order Re2. The kinematic condition to order Re2 yields

H1T=W0T,1. (55)

Substituting (54) evaluated at η=1  into (55) and intergrating with respect to time gives the required order Re2 correction for the film thickness:

H1=-23T-4m>01λm6e-λm2T-1-8m>01λm4e-λm2TT+1λm2-1λm2 +32m>01λmn>0Anm02λn2-λm212λn2e-2λn2T-1-1λm2e-λm2T-1+64m>01λmp>0n>pAnmpλp2+λn2-λm21λp2+λn2e-λp2+λn2T-1-1λm2e-λm2T-1. (56)

To determine the constants C0 and C1  in (36) we introduce an intermediate time scale S=Re2αT, with 0<α<1,  and then match up the expansions for H  and H in their region of overlap (Nayfeh15). Requiring that the coefficients for each power of Re be equal yield C0=3, and

C1=17105+12m>01λm6+32m>01λmn>0Anm02λn2λm2+64m>01λmp>0n>pAnmpλm2λp2+λn2 (57)

The infinite sequence on the right side of (57) converges rapidly to a value of 0.66087, and hence C1=0.8227.

Finally, we can determine a single composite uniform expansion for the transient film thickness by adding the long – time – scale solution (36) to the short – time – scale solution (56) and subtracting from the result their common part:

Hcτ;Re=H0τ+23τ+Re2H1τ/Re2+H1τ-0.6609. (58)

In arriving at (58) we have expressed the short – time – scale contribution H1T to the film thickness in terms of τ.

# IV. Short – time analysis for arbitrary Re

The underlying assumption in Sec. III in developing the short-time solution is that the effects of viscosity are felt throughout the film on spinup. At low Re this is a reasonable assumption. However, at large Re we would anticipate a boundary layer to form at the disk surface in which the effects of viscosity are felt. The boundary layer thus provides a pathway for liquid to leave the disk during spinup, with liquid being supplied to it from an overlying inviscid layer. It is instructive to determine how the fluid layer thins under these circumstances. The problem is similar to the short – time solution for arbitrary Re of Lawrence et al., except that there is an additional equation for the azimuthal component of velocity.

To analyze the boundary layer growth at the disk surface, we follow the procedure detailed in Ref. 10. We suppose that the characteristic time scale tc for the flow is ϵ/Ω, where ϵ is a small constant. Introducing tc=ϵ/Ω into (19), the boundary value problem for F, G, W and H becomes

2F+Wξ=0, (59a)

ReFτ+F2+WFξ-ϵ2ReG2=ϵFξξ, (59b)

ReGτ+2FG+WGξ=ϵGξξ, (59c)

with boundary conditions

Fτ,0=0,       Gτ,0=1,       Wτ,0=0,

Fξτ,H=0,      Gξτ,H=0,     Hτ=Wτ,H, (60)

and initial conditions

F0,ξ=0,         G0,ξ=0,        W0,ξ=0,        H0=1. (61)

To keep the notation simple, we have used the same symbols for the dependent and independent variable as in (21).

Since the effects of viscocity are confined to the boundary layer, there is no azimuthal or radial component of velocity in the overlying inviscid layer. The required solution to (59) as ϵ→0, holding ξ, Re fixed, is thus

Wτ,ξ;ϵ~Δ1ϵa1τ+Δ2ϵa2τ+…. (62)

The function a1τ and a2τ and the scalings Δ1ϵ and Δ2ϵ are to be determined by matching (62) with inner solution that satisfies the boundary conditions at the disk surface. To reflect the presence of a boundary layer, ξ is replaced by a stretched coordinates ζ=ξ/ϵ1/2, and new scalings for F and W are introduced:

F=F/ϵ2,      W=W/ϵ5/2. (63)

It is not necessary to scale G as it is of order unity in the boundary layer; thus G=G. In terms of the inner variables, (59) becomes

2F+Wζ=0,

ReFτ+ϵ2F2+ϵ2WFζ-ReG2=Fζζ,

ReGτ+ϵ2FG+ϵ2WGζ=Gζζ. (64)

In the limit ϵ→0, the leading – order terms satisfy

2F+Wζ=0,        ReGτ=Gζζ,      ReFτ-G2=Fζζ (65)

The boundary conditions at the disk surface are

F0,τ=0,       W0,τ=0,       G0,τ=1. (66)

Since F and G must match with their respective components in the inviscid layer, we have

Fζ,τ→0,    Gζ,τ→0, as ζ→∞ (67)

To solve (65) – (67) we introduce the similarity variable δ=ζRe/4τ1/2 and let G=gδ,   F=τfδ,    W=4τ3/2wδRe1/2. Equations (65) become

g''+2δg'=0,    f''+2δf'-4f=-4g2,    w'=-f. (68)

The boundary conditions for the above set are

f0=0,    f∞=0,   g0=1,

g∞=0,      w0=0. (69)

The prime denotes differentiation with respect to δ. These equations are idential to Benton’s leading – order set that he derived to describe flow due to rotating disk started impulsively from rest, the time – dependent  analog of von Karman’s problem. Closed form for f, g and w are given bay Nigam (see also Benton). The expressions for G, F, and w in terms of the similarity variable δ are

Gζ,τ=erfc δ,

Fζ,τ=2τπ1+2δ2erfc δ-2π-12δe-δ2-2τδ erfc δ-π-12e-δ22,

Wζ,τ=-8τ323πRe123δ+2δ3erfc δ-2π-121+δ2e-δ2+8τ323Re12δδ erfc δ-π-12e-δ22+8τ323πRe12e-δ2erfc δ-8τ3232πRe12erfc 212δ-8τ323τRe122π-1-212+1,(70)

Where erfc is the complementary error function. Matching wζ,τ with the axial velocity (62) in the inviscid layer gives

Δ1ϵ=ϵ52,         a1τ=-8τ323πRe122π-1-212+1. (71)

In order to determine Δ2(ϵ) and a2(τ) in (62), it is necessary to compute F, G and W to order ϵ2. Because the governing equations at this order are complicated by the presence of inhomogeneous terms, it appears that matching with the outer solution is best accomplished numerically. We have not explored this possibility.

To determine the shape of the free surface we let

Hτ,ϵ~1+ϵ52H1τ (72)

And from the kinematic condition we find

H1τ=-16τ5215πRe122π-1-212+1. (73)

Thus the free surface thins according to

Hτ~1-1615πRe122π-1-212+1τ52. (74)

The artificial parameter ha been removed in (7) by taking tc=1/Ω. The result given in holds for arbitrary Reynolds number but obviously becomes invalid when nonlinear effects become important, or when the boundary layer thickness becomes comparable to that of the film such that viscous effects dominate throughout the film.

Benton showed by calculating higher – order corrections to (70) that the first – order approximation becomes inaccurate when τ~0.5. Homsy and Hudson analyzed Benton’s problem numerically and presented results for the entire history of the trasient. From their results we can estimate that the thickness of the viscous region at τ=0.5 is ξ~2.5Re-12. Setting H0.5=2.5Re-1/2 in (74), it then follows that the viscous region is of the film thickness when Re~6.4. This calculation shows that at high Re the short – time solution for the film thickness (74) becomes invalid long before the boundary layer has reached the free surface.

# V. Discussion

The asymptotic expessions for the transient film thickness derived in Sec. III are displayed in Fig. 2. For the purpose of comparision we have chosen the long – time – scale τ as the independent variable for all the expansions, and have taken Re=0.5 t to be a representative value for low Reynolds number flows.

The zero – order term H0τ is shown in Fig. 2 as the dashed curve A. This plot is identical to what Emslie et al. found for the film thickness when they took the initial height of the fluid layer to be independendt of the coordinate r. The two – term long – time – scale solution is shown as the upper  dashed curve B. As expected for short times (τ<0.2 for the case Re=0.5) this expansion becomes singular and predicts a film thichness greater than unity. As noted previously the singular nature of the long – time – scale expansion occurs because the velocity field for the long – time – scale solution does not satisfy the initial conditions. Interesingly, the second – order correction H1τ is always positive, hence H0τ represents a lower bound for the trasient film thickness. It should be noted that the choice of τ as the independent variable in Fig. 2 suppresses the Reynolds number dependence of H0. When this dependence is made explicit, one obtains the expected result that with decreasing Reynolds number, the relative resistance of the film to thinning increases, and a longer spinning time is required to achieve a given reduction in the initial film thickness. As expected the difference between curve A , the lower bound for Hτ, and curve B diminishes with increasing time, and with decresing Reynolds number. For example, at Re=0.5 and τ=0.5 curve B predicts a film thichness about 11% higher than that predicted by H0. At τ=3 the difference in predicted film thickness is about 3%.

The composite uniform expansion given by (58) (curve in Fig. 2) merges with the two – term long – time – scale expansion around τ=0.5, and becomes essentially indistinguishable from it for τ>0.6. For τ<0.5 transient effects due to the initial acceleration of the disk become important, and these effects are accounted  for in the composite expansion through the short – time – scale solution, curve C in Fig. 2.

The composite expansion predicts a film of zero thickness when the spinning time becomes indefinitely large, a result that is obviously physically unreasoable. Not accounted for in this analysis are interfacial effects that eventually dominate the hydrodynamics at the rim of the disk when the film becomes sufficiently thin. In particular, contact line forces will prevent any liquid from being sloughed off the disk, thus enabling the liquid film ro reach gyrostatic equilibrium. When solvent is present evaporation effects must be taken into account to determine the final film thickness.

The pressure distribution in the thinning film follows directly form (13). Substituting the long – time – scale variables and making the pressure dimentionsless with results in

P0τ,ξ;Re=Wξ+WξH-Re2W22-W22H+ξHWτdξ. (75)

The zero – order term for the pressure distribution is thus

P0τ,ξ=W0ξ+W0ξH=ξ2-2H0ξ-H02. (76)

In Fig. 3 we have plotted the pressure distribution, scaled with H2, against ξ/H. Similar plots are also shown for G0, F0, W0 and G1. (Each varialbe has been scaled with an approprivate power of H to make it independent of H when plotted against ξ/H.)

It is evildent from that the pressure distribution at zero order is determined solely by mormal viscous stresses induced by the thinning film and as Fig. 3 shows, the pressure in the film incresases monotonically with distance from the interface, attaining its maximum value ats the disk surface. In order to satisfy the normal stress boundary condition, the pressure at the interface of the thinning film musts be negative with respect to ambient.

A view of some of the hydrodynamic aspects opf centrifugal spinning of liquids may be gained by inspection of the velocity distributions plotted in Fig. 3. It is seen that the velosity field in the thinning film is dominated by the azimuthal component G0, which has a magnitude of unity throughtout the fluid layer. In comparison the radial component of velocity, F0 has a magnitude of order H2 and is parabolic in ξ, the latter feature being typical of lubrication flows. The axial component  W0 is even smaller still, having a magnitude of order H3. Because the first – order correction G1 is negative for all values of ξ, the azimuthal surface velocity of the film lags the angular velocity of disk by 5Re2H4/12.

# VI. Concluding remarks

In this paper we have derived an asymptotic solution that discribes the thinning of a fluid layer on a rotating disk when the Reynolds number for the flow is small. The solution results from matching a long – time – scale expansion that ignores the initial acceleration of the fluid inertia. The leading term in the long – time – scale solution is identical to the result obtained by Emslie et al., and is shown to be a lower bound for the transient film thickness for all τ>0. Although Emslie et al. assumed a uniform pressure throughout the thinning film, the regorous asymptotic analysis shows that there is an axial pressure distribution at zero order, which is induced by normal viscous stresses.

Finally, it should be pointed out that theory is based on the assumption that the interface remains flat during spinning. Consistent with this assumption is the property that the radial and asimuthat velocities depend linearly on the radial coordinate, which allows the problem to be formulated as an exct solution to the time – dependent Navier – Stokes equations, in a manner quite akin to von Karman’s formulation of the problem of a disk rorating in a large quiescent body of fluid. Althrough Emslie et al. have shown that a flat interface is the preferred shape during spinning at low Re, the approximation may well be inappropriate at high Re. Circumferential waves and helical waves are known to occur, though a detailed analysis of their dynamics has not been undertaken. Since the asymptotic analysis present in Sec. IV for arbitrary Re quickly becomes invalid at high Re due to nonlinear effects, it seems fruitful to solve the governing equations presented in Sec. II numerically to gain further insight into film dynamics at high Re. This possibility is presently under investigation.

# Acknowledgments

The author is grateful to S. A. Chen and T. Rehg for material assistance and helpful discussions.

- G.   Emslie, F. J. Bonner, and L. G. peck, J. Appl. Phys. 29,858 (1958).

S. K. Ghandi, VLSI Fabrication Principles (Wiley, New York, 1983).

R. D. Weiss, IEEE Trans. Vol. Mag. 19,1677 (1983).

I. M. Thomas, submitted to Appl. Opt.

A. Acrivos, M. G. Shah, and E. E. Petersen, J. Appl. Phys. 31,963 (1960).

G. F. Damon, Proceedings of the Kodak Seminar on Microminiatureization (Estman Kodak, Roschester, NY, 1969), Vol. 34, P. 195.

B. D. Washo, IBM J. Res. Dev. 21,190 (1977).

D. Meyerhofer, J. Appl. Phys. 49, 3393 (1978).

S. Weinbaum, C. J. Lawrence, and Y. Kuang, J. Fluid Mech. 156, 463 (1985).

C. J. Lawrence, Y. Kuang, and S. Weinbaum, J. Fluid Mech. 156, 479 (1985).

E. R. Benton, J. Fluid Mech. 24, 781 (1966).

H. P. Greenspan and L. N. Howard, J. Fluid Mech. 17, 385 (1963).

T. von Karman, Z. Angew. Math. Mech. 1, 233 (1921).

D. D. Joseph, Arch. Rat. Mech. Anal. 51, 295 (1973).

A. H. Nayfeh, Introduction to Perturbation Techniques (Wiley, New York, 1981).

S. D. Nigam, Quart. Appl. Math. 9,89 (1951).

G. M. Homsy and J. L. Hudson, Appl. Sci. Res. 18, 384 (1968).

H. Espig and R. Hoyle, J. Fluid Mech. 22, 671 (1965)

The mechanics of spin coating of polymer films

C. J. Lawrence

Citation: The Physics of Fluids 31,2786 (1988); doi: 10.1063/1.866986

View Table of Contents: http:// aip. scitation.org/toc/pfl/31/10

Published by the American Institute of Physics

# Articles you may be interested in

- Spin coating: One – dimensional model

Journal of Applied Physics 66, 5185 (1989); 10.1063/1.343754

- A mathematical model for spin coating of polymer resists Journal of Applied Physics 56, 1199 (1984); 10.1063/1.334049

- Flow of a Viscous Liquid on a Rotating Disk Journal of Applied Physics 29, 858 (1958), 10.1063/1.1723300

- Characteristics of resist films produced by spinning Journal of Applied Physics 49,3993 (1978); 10.1063/1.325357

- Dynamics of polymer film formation during spin coating Journal of Applied Physics 116, 123513 (2014), 10.1063/1.4896674

- Theoretical and numerical results for spin coating of viscous liquids Physics of Fluids 16,569 (2004); 10.1063/1.1637353